# DEPLOY VISION LANGUAGE MODEL TO REAL WORLD

# A Senior Project Report

# presented to

# the Faculty of California Polytechnic State University

# San Luis Obispo

# In Partial Fulfillment

# Of the Requirements for the Degree

# Bachelor of Science in Computer Engineering

# By

# DUY NGUYEN

# June 2026

# Advised by Dr. Javier Gonzalez-Sanchez

# Abstract

The intention of this project was to demonstrate the ability to train, fine-tune, and deploy a vision-language-action model in a real-world robotics scenario using Cal Poly infrastructure. The original plan was to collect data in NVIDIA simulation, train, or fine-tune the model inside the NVIDIA simulated environment and then use the resulting policy as a benchmark for real robot deployment. However, this plan was discarded because useful fine-tuning requires the simulation setup to closely match the physical robot setup, camera viewpoint, workspace geometry, and object distribution. Without that match, the simulated data would not provide a reliable benchmark for the physical UR3e system.

After discarding the NVIDIA simulation plan, the project shifted toward fine-tuning Open-VLA directly through Cal Poly infrastructure. This second plan was also not completed because the available driver versions and environment dependencies on the Cal Poly machine were too old for the required Open-VLA fine-tuning setup. As a result, the final implementation focused on raw inference with the pretrained Open-VLA model connected to the UR3e robot. The raw inference pipeline successfully connected camera input, model inference, action conversion, and robot motion, but it achieved 0 percent task success on the physical robot because the model was not fine-tuned to the Cal Poly UR3e environment.

This final report does not provide a meaningful performance benchmark of Open-VLA. Instead, it provides valuable implementation information for future benchmark work. The project documents the infrastructure limitations, deployment path, and robot-inference integration needed before a reliable benchmark can be performed. Future work should use a teleoperation system that integrates data collection with fine-tuning so Open-VLA can be trained on demonstrations from the exact physical setup before success-rate benchmarking.

- Project OverviewProject Overview

A. Topic and Domain

The project is situated at the intersection of robotics, machine learning, and computer vision. The robotics component uses a Universal Robots UR3e arm and RTDE-based Python library controller to control the UR3e. The machine learning component uses Open-VLA - a VLA policy that accepts image and language instruction and predicts the next robot action. The deployment component addresses the problem of running a large open-source model on a limited GPU-local computer. This problem is addressed by using a remote compute while coordinating data exchange with a local camera and robot controller.

B. Goals and Objectives

- The project initially attempted to benchmark Open-VLA by collecting data, training or fine-tuning the model, and then deploying it on the UR3e robot. The first intended approach was to use NVIDIA simulation for both data collection and model training. This approach was discarded because the NVIDIA simulation environment would need to closely match the physical robot environment in order for the fine-tuned model to transfer meaningfully to the real UR3e setup. The project then shifted toward fine-tuning Open-VLA using Cal Poly infrastructure, but this was not possible because the available driver and software environment dependencies were too old for the required fine-tuning setup.

- Because both fine-tuning paths were blocked, the final objective changed from benchmarking a fine-tuned model to validating the raw inference and deployment pipeline. The main objectives were:

- • Set up and deploy the openvla/openvla-7b model on Cal Poly remote compute when possible.
• Build a client/server communication path for sending camera observations to the model and returning predicted actions.
• Integrate OpenVLA action outputs with UR3e robot pose control through the RTDE Python interfaces.
• Test raw inference on the physical UR3e setup and document the result.
• Document why NVIDIA simulation and Cal Poly fine-tuning were not completed, so future teams can avoid repeating the same setup blockers.
• Provide a foundation for future benchmarking after a teleoperation-based data collection and fine-tuning pipeline exists.

C. Intended Users

The intended users are Cal Poly students and developers that want to work in AI robotic integration, manipulation, foundation models, and GPU-based deployment. A future user should be able to reuse this report to understand what components are required, how the inference and robot-control loops are structured, and what engineering issues must be resolved before collecting fine-tuning data or running controlled trials.

D. Relevance and Contributions

LLM has demonstrated its huge capabilities to process natural language. LLM has an advanced intelligent workforce to a magnitude of efficiency. Result in the traumatic increase of need in integration between Neural networks architecture and robotics. This project's first intention attempts to experience the capability of collecting data for VLA, training VLA, and performing inference VLA. This possibility must include the capability of Cal Poly infrastructure.

This project’s first intention was to explore the full workflow for a VLA system: collecting data, training or fine-tuning the model, and performing inference on a physical robot. During development, the project revealed that the hardest part of benchmarking Open-VLA was not only running inference but creating a fine-tuning setup that matched the physical environment and was supported by the available compute infrastructure. The NVIDIA simulation approach was not continued because the simulated setup did not provide a sufficiently matched replacement for the real UR3e environment. The Cal Poly fine-tuning approach was also blocked by outdated drivers and dependency constraints. Therefore, this project contributes a realistic deployment record: it shows that raw OpenVLA inference can be connected to the UR3e pipeline, but also shows that meaningful benchmarking requires more time, newer infrastructure, and a data collection system designed specifically for fine-tuning.

- Background

A. Core Technologies Framework and Usage

These frameworks are necessary to deploy Open-VLA and UR3e Robotic arm. Missing 1 framework will not guarantee to work since the whole project depends on each of these frameworks.

| Tool or Library | Role in the Project |
| --- | --- |
| Python | Primary programing language that implementation model query, camera capture, and robot control. |
| PyTorch | Deep learning framwork used by the OpenVLA model. |
| Hugging Face Transformers | Platform that host Open-VLA. Easy to download and use the model via AutoProcessor() and AutoModelForVision2Seq() for the OpenVLA checkpoint. |
| OpenCV | Captures and resizes camera frames before inference Open-VLA input. |
| Pillow | Converts NumPy image arrays into image objects for the processor. |
| NumPy | Foundation framework for Pytoch and OpenCV. Represents images, actions, pose vectors, and serialized arrays. |
| scipy.spatial.transform | Converts between Euler deltas and UR rotation-vector orientation. |
| RTDE Control/Receive | Sends motion commands to the UR arm and reads current robot state. |
| Fast API / socket | Supports REST-style communication between client and inference process. |

B. Tools and Development Environment

Since the model depends on the framework version and GPU environment. We will use Docker to set up the environment for the server. Manually install dependencies is possible but not recommended since these frameworks are version depended and Open-VLA model needs to depend on. It is best to have an isolated environment to work in.
	A major development limitation was environment compatibility. The original plan expected NVIDIA simulation to support data collection and training, but the simulation setup would have needed to reproduce the same robot, camera angle, workspace, and object distribution as the physical UR3e system. Without that match, fine-tuning on simulated data would not produce a fair physical benchmark. The second plan was to fine-tune OpenVLA directly on Cal Poly infrastructure, but this was blocked because the available driver and dependency versions were too old for the required OpenVLA fine-tuning environment. For this reason, the final implementation used the available environment for raw inference and robot integration rather than model fine-tuning.

- Related Work

This project builds on recent research (2024) in VLA models which are built from LLM backbone and visual encoder [1]. The most relevant systems mentioned in the paper [1] are Open-VLA, RT-2, Open X-Embodiment, and traditional scripted robot automation.

3.1 Open-VLA

Open-VLA is the primary model used in this project. It was designed to be an open alternative to closed generalist robot policies and was trained on broad robot demonstration data. Its relevance data was trained on Open X-Embodiment. Therefore, it has been embedded with generalized data that are useful later if finetuning.

3.2 RT-2

RT-2 is an earlier vision-language-action system developed by Google [2] that demonstrates how web-scale vision-language models could be adapted for robotic control. RT-2 is an important background because it helped establish the VLA formulation: language and perception are not only used for description but are directly mapped to robot actions. However, RT-2-style systems are less accessible to undergraduate deployment projects when model weights and training infrastructure are not openly available.

3.3 Open X-Embodiment

Open X-Embodiment is a large effort to aggregate robot learning datasets across different robot embodiments and tasks [3]. This type of dataset is important because VLA models require broad demonstration data to generalize beyond a single robot and a single lab setup. Open-VLA relies on this diversity data ecosystem, which makes it a reasonable candidate for transfer and future fine-tuning on the Cal Poly UR3e setup.

3.4 Hard-Coded Robot Automation

Traditional robot automation often uses fixed coordinates, predefined waypoints, or object-specific perception pipelines. These methods can be reliable in controlled conditions but tend to fail when the physical scene changes. The goal of this project is not to replace all classical control methods, but to show that a language-and-vision-conditioned policy can provide a more adaptable layer above low-level robot control.

3.5 Gap Addressed by This Project

Existing VLA papers report model-level results, while robot documentation explains low-level control interfaces. The gap for an undergraduate robotics lab is the deployment bridge between the two: how to take an open-source VLA, run it on available GPU infrastructure, feed it live images, convert its action representation into a UR pose command (this happen on the high compute computer cloud that Cal Poly has), evaluated Cal Poly infrastructure to host and deployment.

- Features, Requirements, and Evaluation Criteria

A. Key Features

- Image-conditioned from live camera frames.

- Remote-machine deployment of Open-VLA with GPU-aware model loading.

- Client/server communication for sending observations and receiving actions.

- Conversion of Open-VLA action deltas into UR TCP pose targets.

- Safety stop monitoring through the robot receive interface.

B. Functional Requirements

| Functional Requirement | Description | Implementation Evidence |
| --- | --- | --- |
| Live Camera Capture | Camera that connects to client and can capture a camera frame in a loop | The client code uses OpenCV. Server uses OpenCV headless. VideoCapture and resize frames before sending or displaying them. |
| Robotic arm | 6 DoF robotic arm or 7 Dof (if includes griper). | The client invokes moveL() to the robot which takes 6 arguments required to control robot. |
| Host | A computer | Inference into high parameter VLA model |

C. User Interaction Requirements

The current prototype is developer-facing rather than end-user-facing. A user starts the inference server. Then start the ssh tunnel. Then starts the client connected to the robot and camera and observes the robot's behavior. The project does not include a graphical user interface.

D. Non-Functional Requirements

| Category | Requirement |
| --- | --- |
| Compute | The model-serving process should use CUDA when available and will fall back to CPU if CUDA is not available. |
| Safety | Robot motion must be scaled and monitored to reduce the chance of unsafe movements. Protective stops must terminate on. |

E. Evaluation Criteria

The evaluation criteria focus on deployment of feasibility (latency) and integration rather than performance benchmark (success rate). A future project should add quantitative benchmarks after fine-tuning.

- System Design

A. System Architecture and Components

The system is organized as a distributed robot-control pipeline. The camera and robot-control code run near the robot. The Open-VLA inference process runs on a machine capable of loading the model, preferably a GPU-backed remote machine. Communication between the two sides transmits image observations to the model and returns action vectors to the robot-control process.

Figure 1. High-level deployment architecture for Open-VLA-assisted UR3e control.

B. Algorithms and Processing Logic

The main processing logic follows a repeated sense-infer-act loop. Each cycle begins with the camera observing the workspace and ends with the robot moving toward a target pose predicted from the model output.

The processing workflow is:

- Capture Image Frame
The client opens the camera using OpenCV and continuously reads image frames. Each frame is resized before being sent to the model server so that the input image size is consistent with the model's pipeline.

- Send Image and Instruction
The image is paired with a language instruction such as “pick up the orange object in the middle of the table.” This combination of visual input and task instruction is sent to the Open-VLA inference endpoint.

- Run Open-VLA Inference
The model predicts an action using predict_action. The implementation uses inference mode, so gradients are not computed, which reduces memory usage and improves runtime efficiency.

- Return Action Vector
The predicted action is returned to the client as a numerical array. The action contains movement deltas that represent how the robot should move relative to its current state.

- Read Current Robot Pose
Before moving the robot, the client reads the current TCP pose from the UR3e using the RTDE receive interface.

- Convert OpenVLA Action to UR Pose
The first three values of the action are interpreted as translational deltas: dx, dy, and dz. These values are scaled and added to the current TCP position. The next three values are interpreted as rotational deltas: roll, pitch, and yaw. These are converted into a rotation object and combined with the current UR rotation vector. The resulting orientation is converted back into a UR rotation vector.

- Execute Robot Motion
The converted target pose is sent to the UR3e using a linear motion command. After each motion, the system checks whether the robot has entered a protective stop state. If so, execution stops.

The core algorithm can be summarized as:

while system is running:
    capture camera image
    send image and instruction to OpenVLA server
    receive predicted action vector
    read current UR3e TCP pose
    convert action vector into UR target pose
    command UR3e to move to target pose
    check for protective stop

This processing loop demonstrates the central idea of the project: replacing hardcoded object-specific motion logic with a more adaptive vision-language-action model that can reason from visual input and task instructions.

C. Interfaces and APIs

The interface that the client is communicating with model is /act fastapi. The uploaded code includes a REST-style client path that sends image and instruction data to an /act endpoint, as well as a custom TCP server implementation that serializes NumPy arrays and sends them through a socket connection if client prefer TCP rather fastapi. The TCP server accepts a client connection, receives an image encoded as a NumPy array, runs Open-VLA inference, and sends a NumPy action array back to the client. TCP does not support client language instruction.

E. Implementation Details

The implementation is organized into separate source files so that each part of the system has a clear responsibility. This organization also makes the system easier to test and modify because camera capture, model inference, network communication, and robot motion are not all placed in one file.

The major source files are organized as follows:

openvla-client.py: Client-side control loop for camera capture, model request, and 	       robot motion.

ur_robot.py: UR3e robot wrapper and OpenVLA-action-to-UR-pose conversion.

main.py: OpenVLA inference server setup and inference loop.

server.py: Lightweight TCP server for NumPy image and action transfer.

The openvla-client.py file implements the local robot-side control loop. It connects to the UR3e robot, opens the camera using OpenCV, captures image frames, resizes the frames, sends the visual observation and task instruction to the OpenVLA inference endpoint, receives a predicted action, reads the current robot TCP pose, converts the action into a UR-compatible target pose, and sends the movement command to the robot. This file acts as the main coordinator between the physical camera, OpenVLA model output, and robot execution.

The ur_robot.py file defines the ur_robot class. This class wraps the Universal Robots RTDE control and receive interfaces. The RTDE receive interface is used to read the robot’s current TCP pose and protective stop status, while the RTDE control interface is used to send motion commands. The most important function in this file is convert_openvla_to_ur_pose, which translates the OpenVLA action vector into a UR3e target pose. This conversion is necessary because OpenVLA does not directly output a complete UR3e moveL command.

The main.py file implements the remote OpenVLA inference process. It creates the Open-VLA server, loads the openvla/openvla-7b model using Hugging Face AutoProcessor and AutoModelForVision2Seq to auto download the model weight, selects the compute device, configures the model data type, receives image observations, prepares the model input, runs inference with predict_action, and sends the predicted action back to the client. The code checks whether CUDA is available and uses GPU inference when possible. If a compatible GPU is available, the program selects either bfloat16 or float16 precision depending on GPU capability.

The server.py file implements a lightweight TCP communication server. This is optional if the user doesn’t want to use fastapi. It creates a socket, listens to a client's connection, receives serialized NumPy image arrays, decodes the arrays, and sends serialized NumPy action arrays back to the client. This file provides the communication layer between the local robot-side client and the remote inference machine.

- Testing, Experiments, and Evaluation

A. Testing Approach

The evaluation was structured as integration testing rather than formal machine-learning benchmarking. The original benchmark plan required a fine-tuned model, but fine-tuning was not completed because the NVIDIA simulation setup did not sufficiently match the physical setup, and the Cal Poly infrastructure had outdated driver and environment dependencies. Therefore, testing focused on whether the raw pretrained OpenVLA model could be connected to the physical robot pipeline. The tested stages were model loading, image capture, image transfer, action prediction, action conversion, and robot command generation. Because Open[VLA was not fine-tuned on the Cal Poly UR3e setup, the task success rate was expected to be poor and was not treated as a model benchmark.

B. Experimental Setup

The experimental setup represents the physical target environment that future fine-tuning should match. This is the reason the NVIDIA simulation plan was discarded: a useful simulated training environment would need to reproduce the same UR3e embodiment, camera viewpoint, table layout, object placement, and action scaling. Since that match was not completed, the project moved away from simulation-based training. The physical experiment therefore used the real UR3e, webcam, tabletop objects, router connection, and remote inference machine to test raw model deployment rather than fine-tuned policy performance.

C. Results

The main result is that the project produced an end-to-end implementation path for using Open-VLA as a robot action policy in the Cal Poly environment. The code demonstrates that camera frames can be captured, model inference can be invoked, actions can be returned, and action vectors can be transformed into UR pose targets. However, the physical robot experiment using raw pretrained Open-VLA inference achieved 0 percent task success. This result should not be interpreted as a final benchmark of Open-VLA because the model was not fine-tuned on the target robot, camera viewpoint, workspace geometry, object set, or action scaling. Instead, the result shows that raw inference alone is not sufficient for reliable UR3e manipulation in this setup. The validated outcome of the project is the deployment and integration pipeline, while successful physical manipulation remains future work.

D. Evaluation Against Requirements

The system partially satisfies the functional requirements defined earlier in the report. Live camera capture was achieved through OpenCV, and the client was able to capture and resize frames for model input. The robotic arm requirement was partially satisfied because the UR3e could receive RTDE commands generated from Open-VLA action outputs. The host requirement was also partially satisfied because the model could be served for raw inference, although the infrastructure was not sufficient for fine-tuning due to outdated driver and dependency constraints.

The system satisfies the integration goal but does not satisfy the original benchmark goal. The project successfully connected the major components of the pipeline: camera input, language instruction, remote model inference, action-vector transfer, UR pose conversion, and robot motion. However, the raw pretrained model produced 0 percent physical task success. This confirms that future evaluation must be performed only after collecting demonstrations and fine-tuning the model on the exact physical setup. A meaningful benchmark should compare scripted automation, raw Open-VLA, and fine-tuned Open-VLA using repeated trials, task success rate, latency, object placement variation, and failure-mode analysis.

- Future Work

The most important next step is fine-tuning OpenVLA on demonstrations collected from the UR3e robot and the exact Cal Poly tabletop environment. Fine-tuning would align the model with the robot embodiment, camera viewpoint, workspace geometry, object types, and action scaling used in the lab. After fine-tuning, the system should be evaluated with a controlled benchmark that records success rate, number of trials, object placement variations, inference latency, and failure modes.

- Collect a small UR3e demonstration dataset for common pick, place, and reach tasks.

- Fine-tune OpenVLA

- Implement workspace safety constraints, maximum action deltas, joint-limit checks, and emergency-stop procedures.

- Add gripper control and evaluate full pick-and-place tasks rather than arm-only motion.

- Create a benchmark comparing scripted automation, un-fine-tuned OpenVLA, and fine-tuned OpenVLA under scene variations.

- Measure success rate benchmarks as the model improves after finetunning.

- Conclusions

This senior project demonstrated the feasibility of deploying an open-source vision-language-action model in a real robotics workflow using Cal Poly remote compute and a UR3e robot-control stack. The project connected camera input, language instructions, OpenVLA inference, action conversion, and RTDE robot motion into a proof-of-concept pipeline. The work shows that OpenVLA can be treated as more than an offline research artifact: it can be loaded, served, queried, and connected to physical robot control code in an undergraduate engineering environment.

The project has not achieved the ability to provide a reliable way to collect data for model finetuning like teleoperating with the robot.

The current implementation does not provide a meaningful task success benchmark. As it has not achived any success on any task that was given for. And it should not be treated as a reliable autonomous manipulation system without fine-tuning and stronger safety controls. Its value is that it reduces setup uncertainty for future work. Future teams can build on the documented architecture, code organization, deployment assumptions, and limitations to fine-tune the model for the UR3e and evaluate whether VLA-based control is more adaptable than hard-coded automation under realistic environmental variation.

- References

[1] M. J. Kim et al., "OpenVLA: An Open-Source Vision-Language-Action Model," Proceedings of Machine Learning Research, 2025. Also available as arXiv:2406.09246, 2024.

[2] A. Zitkovich et al., "RT-2: Vision-Language-Action Models Transfer Web Knowledge to Robotic Control," Conference on Robot Learning, 2023.

[3] Open X-Embodiment Collaboration et al., "Open X-Embodiment: Robotic Learning Datasets and RT-X Models," arXiv:2310.08864, 2023.

- Project Schedule

First Quarter

| Week | Tasks | Milestones |
| --- | --- | --- |
| 1-2 | Contact professor Meet up with people who also work on robots. Going on a tour with the robot |  |
| 3 | Getting familiar with Docker, Robot simulation | Docker simulator to run and able to send command. |
| 4-7 | Research to get real-time robot. |  |
| 8-9 | Getting Familiar with Nvidia sim. Set up the physical environment. Able to initialize a physical world with the robotic model. | Finish setting up all environments with the robotic model. Retrieve the stage of the world interaction and robot stages. |
| 10 | Try to deploy simple neural net, LLM,.. Into the Nvidia Sim |  |

Second Quarter

| Week | Tasks | Milestones |
| --- | --- | --- |
| 2 | Research on a few model that works well with robotic arm |  |
| 4-7 | Setting up a data exchange pipeline on server and client. | Integrate the machine learning model into the robotic simulation on docker via the setup pipeline |
| 8-9 | Deploy the data exchange pipeline with real physical environment | Data exchange and model inference success. |
| 10 |  | Finishing report |